EE-2003 Computer Organization and Assembly Language

LAB 02

Getting Started with Assembly Language (Code Composing, Compilation, Creating Output Files)

Course Code: EE-2003

Semester: Fall 2025

Instructor: Engr. Muhammad Qasim

Email: Muhammad.qasim@nu.edu.pk

Department of Computer Science, National University of Computer and Emerging Sciences FAST Peshawar

Objective: The primary objective of this lab is to familiarize students with the fundamental steps of writing, compiling, and running a simple assembly language program using an emulator like DOSBox and the NASM assembler. We will focus on the process of writing code, assembling it into an object file, and finally, running the program.

Part 1: Foundational Concepts from Intel iAPX88 Architecture

The iAPX88 architecture, which is the foundation of the x86 family of processors, is crucial for understanding how our programs work in DOSBox. Here are some key concepts:

Registers

Registers are small, high-speed storage locations within the CPU. We will primarily work with the following types:

- General-Purpose Registers (AX, BX, CX, DX): These are 16-bit registers used for arithmetic and data manipulation. They can also be accessed as two 8-bit registers (e.g., AX is composed of AH and AL).

- Segment Registers (CS, DS, SS, ES): These registers are used to define memory segments, which are 64KB blocks of memory. The Code Segment (CS) holds your program's instructions, the Data Segment (DS) holds your program's data, and the Stack Segment (SS) holds the program's stack.

Instruction Groups

Instructions in assembly language can be categorized into groups based on their function:

- Data Movement: Instructions like “mov” move data between registers and memory.

- Arithmetic: Instructions like add and sub perform mathematical calculations.

- Program Control: Instructions like int manage program execution flow.

### Part 2: Hands-On Exercise - Our First Program

Follow these steps to write, assemble, and run a program that adds two numbers. This is our first program, demonstrating the use of data movement and arithmetic instructions.

Step 1: Composing the Code

- Open a text editor on local machine.

- Type the following code exactly as it is written. This program will add the numbers 5 and 10 and store the result.

[org 0x0100]

; start of code

mov  ax, 5              ; move the constant 5 into register ax

mov  bx, 10

add  ax, bx             ; add value of bx into the value of ax

mov  bx, 15             ; add constant 15 into the value of bx

add  ax, bx

mov  ax, 0x4c00         ; exit ..

int  0x21               ; .. is what the OS should do for me

; watch the listing carefully

- Save the file as ADD.ASM and exit the editor.

Step 2: Assembling the Code

- Use the NASM assembler to convert your .asm file into an executable .com file.

- Type the following command and press Enter:

nasm ADD.ASM -o ADD.COM

- The assembler will directly create the executable file (ADD.COM) if there are no syntax errors in your code.

Step 3: Running the Program

- Now you can execute your program directly from the DOS prompt.

- Type the following command and press Enter:

ADD.COM

- The program will run, perform the addition, and terminate. Since there is no output to the screen, you won't see anything. However, you can use a debugger to verify the result of the addition.

Part 3: Debugging and Verification

Using a debugger allows you to step through the program and inspect the values in registers.

- To start the debugger, type:

afd ADD.COM

- Use the F1 command to execute one instruction at a time and observe the changes in the registers.

- Use the r (register) command to view the current state of all registers.

- After tracing all instructions, you will be able to see that the AX register contains the value 15 (which is 000F in hexadecimal).

- Press q to quit the debugger.

Part 4: Lab Tasks

Complete the following tasks and demonstrate your work to the lab instructor.

- Subtraction Program: Write a new assembly program in DOSBox that subtracts 25 from 50. Assemble and run the program. Use the debugger to prove that your program worked correctly and the result is stored in AX.

- Combined Operations: Write a program that first adds 15 to 10 and then subtracts 5 from the result. Use the debugger to verify that the final value in AX is 20.