EE-2003 Computer Organization and Assembly Language

LAB 05

Important Flags

Course Code: EE-2003

Semester: Fall 2025

Instructor: Engr. Muhammad Qasim

Email: Muhammad.qasim@nu.edu.pk

Department of Computer Science, National University of Computer and Emerging Sciences FAST Peshawar

Lab Manual: Introduction to Processor Flags

Objective

The purpose of this lab is to familiarize students with the processor flags, which are single-bit registers that provide information about the status of the CPU and the result of a mathematical or logical operation. Understanding these flags is crucial for writing efficient assembly language programs, especially for conditional branching and error checking. We will use the AFD (80x86 Debugger) interface to observe how various instructions affect the flag register.

Prerequisites

- Familiarity with the AFD (80x86 Debugger) interface.

- Basic understanding of 80x86 assembly language instructions.

The Flag Register

The flag register is a 16-bit register in the 80x86 architecture. Each bit in this register is a flag with a specific purpose. We will focus on the most commonly used flags.

Image of 8086 flag register:

- OF (Overflow Flag): Set to 1 if the result of a signed arithmetic operation is too large or too small to fit in the destination operand.

- IF (Interrupt Flag): When set to 1, the CPU responds to maskable interrupt requests from external devices. When 0, these interrupts are ignored.

- SF (Sign Flag): Reflects the sign of the result of an operation. It's a copy of the most significant bit (MSB) of the result. If the MSB is 1, the result is negative; if 0, it's positive.

- ZF (Zero Flag): Set to 1 if the result of an arithmetic or logical operation is zero.

- AF (Auxiliary Flag): Used for BCD (Binary Coded Decimal) arithmetic. It's set to 1 if there's a carry or borrow out of bit 3 into bit 4 of an 8-bit operation.

- PF (Parity Flag): Set to 1 if the number of set bits (1s) in the least significant byte of the result is even.

- CF (Carry Flag): Set to 1 if there's a carry out of the most significant bit of the result during an addition, or a borrow during a subtraction. It's often used for multi-precision arithmetic.

Lab Activities

Activity 1: Observing Arithmetic Flags (CF, ZF, SF, OF)

- Open the AFD debugger.

- Use the t (trace) command to execute instructions one at a time and observe how the flags change.

- CF (Carry Flag):

  - Instruction: MOV AX, FFFFh

  - Instruction: ADD AX, 1

  - Observation: The result FFFFh + 1 is 10000h, which doesn't fit in the 16-bit AX register. The AX register becomes 0000h, and the CF is set to 1, indicating a carry out of the most significant bit.

- ZF (Zero Flag):

  - Instruction: MOV AX, 10

  - Instruction: SUB AX, 10

  - Observation: The result is 0. The ZF will be set to 1.

- SF (Sign Flag):

  - Instruction: MOV AX, 10

  - Instruction: SUB AX, 20

  - Observation: The result is 10 - 20 = -10. In two's complement, -10 is FFF6h. Since the most significant bit is 1, the SF is set to 1.

- OF (Overflow Flag):

  - Instruction: MOV AL, 7FH (decimal 127, the max for a signed byte)

  - Instruction: ADD AL, 1

  - Observation: The result is 80h (decimal -128). This is a positive number added to a positive number resulting in a negative number, which is a signed overflow. The OF will be set to 1.

Activity 2: Observing PF and AF

- PF (Parity Flag):

  - Instruction: MOV AL, 0B3h (1011 0011b)

  - Instruction: ADD AL, 0

  - Observation: The number of set bits in B3h is 5 (odd). The PF will be 0.

  - Instruction: MOV AL, 065h (0110 0101b)

  - Instruction: ADD AL, 0

  - Observation: The number of set bits in 65h is 4 (even). The PF will be 1.

- AF (Auxiliary Flag):

  - Instruction: MOV AL, 9

  - Instruction: ADD AL, 8

  - Observation: 9 + 8 is 11h. There is a carry from the lower nibble (9h + 8h = 11h). The AF will be set to 1.

Conclusion

Processor flags are essential for control flow and decision-making in assembly language. By observing how they are affected by different instructions, you can gain a deeper understanding of how the CPU processes data and makes decisions. This knowledge is fundamental for advanced assembly programming and computer architecture concepts.

Task 1: Check the Zero Flag (ZF)

Objective: Write a program to set the Zero Flag (ZF) and then clear it.

Instructions:

- Initialize a register (e.g., AX) to a non-zero value.

- Subtract a value from AX to make the result zero.

- Observe the ZF. It should be set to 1.

- Perform an operation that results in a non-zero value.

- Observe the ZF again. It should be reset to 0.

Solution:

Code snippet

[org 0x0100]

mov ax, 50h

sub ax, 50h   ; Result is 0, ZF is set to 1

mov ax, 10h   ; Result is not 0, ZF is cleared to 0

mov ax, 0x4c00

int 0x21

Task 2: Check the Sign Flag (SF)

Objective: Write a program to set the Sign Flag (SF) by performing a subtraction.

Instructions:

- Initialize a register (e.g., BX) with a value.

- Subtract a larger value from BX to get a negative result.

- Observe the SF. It should be set to 1.

- Perform an operation that results in a positive number.

- Observe the SF. It should be reset to 0.

Task 3: Check the Carry Flag (CF)

Objective: Write a program to set the Carry Flag (CF) by adding a value that overflows the register.

Instructions:

- Initialize an 8-bit register (AL) to its maximum value.

- Add a small value to AL that will cause an overflow.

- Observe the CF. It should be set to 1.

Task 4: Write a code to observe each flag and take screenshot to show the changes in flags.