Program: BS Computer Science

Semester: Fall 2025

Credit Hour(s): 1

Pre-requisite(s): EE1005 Digital Logic Design

Lab: Computer Organization and Assembly Language

Instructor: Engr. Muhammad Qasim – muhammad.qasim@nu.edu.pk

Lab # 8: Rotation and Extended Addition, subtraction and Multiplication

1. Lab Objectives

Upon completion of this lab, you will be able to:

- Understand and implement the basic Rotate instructions: Rotate Left (ROL) and Rotate Right (ROR).

- Understand and implement the Rotate through Carry instructions: Rotate through Carry Left (RCL) and Rotate through Carry Right (RCR).

- Apply RCL and RCR to perform Extended Shifting of multi-word operands, effectively joining two or more words.

- Implement Extended Addition and Extended Subtraction using the Add with Carry (ADC) and Subtract with Borrow (SBB) instructions.

- Understand the necessity of Extended Operations for handling arithmetic results that exceed the native word size of the architecture (e.g., 16-bit)

## 2. Theoretical Concepts and Instructions

The lab focuses on how basic 16-bit operations are chained together to handle larger data sizes (32-bit, 64-bit, etc.), a process referred to as extended operations5.

### 2.1. Rotation Instructions

Rotation instructions move bits around a register or memory location. The bits shifted out of one end are moved back in on the opposite end.

| Instruction | Description | Effect on Carry Flag (CF) |
| --- | --- | --- |
| ROL (Rotate Left) | Rotates the bits of the operand to the left. The bit shifted out of the MSB (Most Significant Bit) is placed into the LSB (Least Significant Bit) and also copied into the Carry Flag (CF). | Copies MSB to CF. |
| ROR (Rotate Right) | Rotates the bits of the operand to the right. The bit shifted out of the LSB is placed into the MSB and also copied into the Carry Flag (CF). | Copies LSB to CF. |
| RCL (Rotate through Carry Left) | Rotates the bits of the operand and the Carry Flag (CF) to the left. The bit shifted out of the MSB goes into CF, and the old value of CF is pushed into the LSB6. | MSB goes to CF. |
| RCR (Rotate through Carry Right) | Rotates the bits of the operand and the Carry Flag (CF) to the right. The bit shifted out of the LSB goes into CF, and the old value of CF is pushed into the MSB. | LSB goes to CF. |

2.2. Extended Operations

Extended operations use the Carry Flag (CF) to link two or more instructions, allowing arithmetic and logical operations on numbers larger than the processor's word size (e.g., 32-bit numbers on a 16-bit processor)7.

| Operation | Instructions Used | Purpose |
| --- | --- | --- |
| Extended Shifting | SHL/SHR and RCL/RCR | To shift a multi-word number. The carry bit generated by shifting the lower word is then rotated into the higher word using RCL or RCR8. |
| Extended Addition | ADD and ADC (Add with Carry) | To add two multi-word numbers. The ADC instruction is specifically designed to add two operands plus the value of the Carry Flag (CF), effectively including any carry from the addition of the previous, lower word9. |
| Extended Subtraction | SUB and SBB (Subtract with Borrow) | To subtract two multi-word numbers. The SBB instruction subtracts the source from the destination and also subtracts the value of the Carry/Borrow Flag (CF). If the lower subtraction needed a borrow, SBB ensures that a '1' is subtracted from the upper half10101010. |
| Extended Multiplication | SHR, SHL, ADD, ADC, RCL | Multiplication of 16-bit operands can produce a 32-bit result. Extended multiplication algorithms iterate through the multiplier's bits, using extended addition (ADD/ADC) to accumulate the 32-bit result and extended shifting (SHL/RCL) to align partial products11111111. |

Lab Tasks

- Write the inversed lower bits of AL into AH using shifting and rotation

- Perform extended multiplication using shifting and rotation.