Program: BS Computer Science

Semester: Fall 2025

Credit Hour(s): 1

Pre-requisite(s): EE1005 Digital Logic Design

Lab: Computer Organization and Assembly Language

Instructor: Engr. Muhammad Qasim – muhammad.qasim@nu.edu.pk

Lab # 12 STRING INSTRUCTIONS & BLOCK PROCESSING

Lab Objectives

By the end of this lab, students will be able to:

- Understand the concept of block processing in assembly language.

- Utilize Source Index (SI) and Destination Index (DI) registers effectively.

- Manipulate the Direction Flag (DF) using CLD and STD instructions.

- Implement string instructions: STOS, LODS, SCAS, MOVS, and CMPS.

- Apply repetition prefixes (REP, REPE, REPNE) to perform loop-like operations in a single instruction.

### Theoretical Background

2.1. String Instructions Overview The 8088 processor provides five specialized instructions for processing blocks of data (strings). These instructions allow for efficient code by performing multiple operations (data movement + pointer update) in a single cycle.

| Instruction | Name | Operation | Typical Use |
| --- | --- | --- | --- |
| STOS | Store String | Store AL/AX at ES:DI | Clearing buffers/screens |
| LODS | Load String | Load from DS:SI to AL/AX | String processing loops |
| SCAS | Scan String | Compare AL/AX with ES:DI | Searching for chars/length |
| MOVS | Move String | Move from DS:SI to ES:DI | Copying memory blocks |
| CMPS | Compare String | Compare DS:SI with ES:DI | Comparing strings |

Registers & Flags

- DS:SI (Source Index): Points to the source string.

- ES:DI (Destination Index): Points to the destination string.

- CX (Counter): Used by REP prefixes to determine the number of iterations.

- DF (Direction Flag):

  - If 0 (Clear): Auto-increment mode (Process low address $\to$ high). Set using CLD.

  - If 1 (Set): Auto-decrement mode (Process high address $\to$ low). Set using STD.

2.3. Repetition Prefixes

- REP: Repeat instruction CX times (Used with STOS, MOVS).

- REPE / REPZ: Repeat while Equal / Zero Flag is Set (Used with CMPS, SCAS).

- REPNE / REPNZ: Repeat while Not Equal / Zero Flag is Clear (Used with CMPS, SCAS).

Exercise 7.1: Clearing the Screen using STOS

Objective: Use STOSW with the REP prefix to fill the video memory efficiently.

Code Logic:

- Set ES to Video Memory segment (0xB800).

- Set DI to 0 (Top-left corner).

- Load AX with the character/attribute to fill (e.g., Space 0x20 with attribute 0x07 $\to$ 0x0720).

- Load CX with the screen size (2000 words).

- Execute REP STOSW.

Task: Write a program to clear the screen with a Blue background and White text. Hint: Attribute byte is 0x17 (Blue background=1, White foreground=7). ASCII for space is 0x20.

Exercise 7.2: Printing a String using LODS and STOS

Objective: Use LODS to fetch data and STOS to display it. Note that LODS is rarely used with REP.

Code Logic:

- Set DS:SI to point to your "Hello World" string.

- Set ES:DI to point to a specific location on the video screen.

- Loop CX times (length of string):

  - LODSB: Load byte from string into AL.

  - MOV AH, 0x07: Attach attribute.

  - STOSW: Store char+attr to video memory.

Task: Modify Example 7.2 from the handouts to print your Name and Roll Number at the center of the screen (Row 12, Column 30).

Exercise 7.3: Calculating String Length using SCAS

Objective: Use SCASB with REPNE to find the length of a null-terminated string.

Code Logic:

- Set ES:DI to point to the string.

- Set AL to 0 (Null terminator).

- Set CX to 0xFFFF (Max scan size).

- Execute REPNE SCASB.

- Calculate length: 0xFFFF - CX - 1.

Task: Write a subroutine strlen that accepts a string address (segment/offset) via the stack, calculates the length, and returns it in the AX register. Use the LES instruction to load the pointer.

Exercise 7.4: Screen Scrolling using MOVS

Objective: Use MOVSW to copy large blocks of memory (scrolling). Understand the importance of processing direction (STD vs. CLD).

Code Logic (Scroll Up):

- Source (SI): Start of line 2 (offset 160).

- Destination (DI): Start of line 1 (offset 0).

- Count (CX): Total words to move.

- Direction: CLD (Forward) because Source > Destination.

Code Logic (Scroll Down):

- Source (SI): End of 2nd last line.

- Destination (DI): End of last line.

- Count (CX): Total words.

- Direction: STD (Backward) to prevent overwriting data before it is copied.

Task: Write a program that scrolls the entire screen down by 5 lines. Ensure the top 5 lines are cleared (black) after the scroll.

Exercise 7.5: String Comparison using CMPS

Objective: Use CMPSB with REPE to compare two strings.

Code Logic:

- Set DS:SI to String 1.

- Set ES:DI to String 2.

- Set CX to the length of the strings.

- Execute REPE CMPSB.

- Check Zero Flag (ZF):

  - If ZF=1: Strings are equal.

  - If ZF=0: Strings are different.

Task: Write a "Password Check" program.

- Define a secret password in memory (e.g., "secret").

- Define an input buffer with a different string (e.g., "secrecy").

- Use CMPS to compare them.

- If they match, print "Access Granted" (Green).

- If they differ, print "Access Denied" (Red).

4. Lab Submission Requirements

- Code Files: Submit .asm files for all 5 tasks.

- Screenshots: Provide screenshots of the output (or debugger state) for each task.

- Analysis: For Task 4 (Scrolling), explain briefly in your own words why STD was required for scrolling down but CLD was used for scrolling up.

5. Challenge Problem

"The Marquee Effect" Write a TSR (Terminate and Stay Resident) program or a standard loop that rotates a text string on the top line of the screen.

- The string should move from right to left.

- When a character disappears from the left (column 0), it should reappear on the right (column 79).

Notes:

The five instructions are STOS, LODS, CMPS, SCAS, and MOVS called store string, load string, compare string, scan string, and move string respectively. MOVS is the instruction that allows memory to memory moves, as was discussed in the exceptions to the memory to memory movement rules.

Firstly they all work on a block of data. DI and SI are used to access memory. SI and DI are called source index and destination index because of string instructions. Whenever an instruction needs a memory source, DS:SI holds the pointer to it. An override is possible that can change the association from DS but the default is DS. Whenever a string instruction needs a memory destination, ES:DI holds the pointer to it.

Whenever a byte register is needed, AL holds the value. Whenever a word register is used AX holds the value. For example STOS stores a register in memory so AL or AX is the register used and ES:DI points to the destination. The LODS instruction loads from memory to register so the source is pointed to by DS:SI and the register used is AL or AX

If DF is cleared, this is called the auto increment mode of string instruction, and if DF is set, this is called the auto decrement mode. There are two instructions to set and clear the direction flag.

STOS transfers a byte or word from register AL or AX to the string element addressed by ES:DI and updates DI to point to the next location. STOS is often used to clear a block of memory or fill it with a constant.

If DF is clear, SI will be incremented by one or two depending of whether STOSB or STOSW is used. If DF is set SI will be decremented by one or two depending of whether STOSB or STOSW is used. If REP is used before this instruction, the process will be repeated CX times.

STOSW: takes value from ax and store at es:di

REP: repeat CX times

CLD: clear direction flag, if this flag is clear then auto increment di and if this flag is set then auto decrement di

Repne: repeat until not equal

Scasb: goes to es:di and compare that value with ax and increment di and cx is decremented if rep is used