Program: BS Computer Science

Semester: Fall 2025

Credit Hour(s): 1

Pre-requisite(s): EE1005 Digital Logic Design

Lab: Computer Organization and Assembly Language

Instructor: Engr. Muhammad Qasim – muhammad.qasim@nu.edu.pk

Lab # 13 INTRODUCTION TO DELAY LOOPS AND TIMING

Objective

- To understand the concept of software-based time delays in 8086/8088 assembly.

- To learn how to calculate precise execution time based on clock cycles.

- To implement nested loops to achieve visible delays (seconds) rather than microseconds.

- To apply delay loops in real-world scenarios like text blinking or animation.

Theory & Background

What is a Delay Loop?

A delay loop is a sequence of instructions designed specifically to "waste" time. The processor executes these instructions (usually decrementing a counter and jumping back) to keep itself busy for a specific duration.

Timing Calculation (The Math)

The duration of the delay depends on two factors:

- CPU Frequency: The speed of the processor (e.g., 4.77 MHz for the original 8088).

- Cycle Count: The number of clock cycles each instruction takes to execute.

Formula:

$$\text{Time} = \text{Total Clock Cycles} \times \text{Period of one Cycle}$$

Example from Lecture:

- Processor: 8086 running at 4.77 MHz.

- Time per cycle: $1 / 4.77\text{ MHz} \approx 210\text{ ns}$.

- To achieve a 1 second delay, you need approximately 4,761,905 cycles.

The Nested Loop Structure

Since a single register (like CX) is 16-bit, it can only count up to 65,535.

- $65,535 \times \text{approx } 10\text{ cycles} = 655,350\text{ cycles}$.

- $655,350 \times 210\text{ ns} \approx 0.13\text{ seconds}$.

To get delays of 1 second or more, we must use Nested Loops (loops inside loops). The total cycles are roughly:

$$\text{Total Cycles} \approx \text{Outer Count} \times \text{Middle Count} \times \text{Inner Count} \times \text{Cycles per Inner Iteration}$$

4. Lab Procedure

Step 1: The Basic Delay Routine

Copy the following standard delay template into a file named delay_test.asm. This uses the logic defined in your lecture slides (DELAY2 routine).

; delay_test.asm

[org 0x0100]

mov si, msg_start   ; Print "Start"

call print_string

call delay_routine  ; Wait...

mov si, msg_end     ; Print "End"

call print_string

mov ax, 0x4c00      ; Terminate

int 0x21

; ------------------------------------------------

; Delay Subroutine (Based on Week 16 Slides)

; ------------------------------------------------

delay_routine:

push cx

push bx

push dx

mov cx, 100         ; Outer Loop Counter

outer_loop:

mov bx, 200         ; Middle Loop Counter

middle_loop:

mov dx, 200         ; Inner Loop Counter

inner_loop:

dec dx              ; 2 cycles

jnz inner_loop      ; 16 cycles (taken) / 4 (not taken)

dec bx

jnz middle_loop

dec cx

jnz outer_loop

pop dx

pop bx

pop cx

ret

; ------------------------------------------------

; Helper: Print String

; ------------------------------------------------

print_string:

lodsb

cmp al, 0

je done_print

mov ah, 0x0E

int 0x10

jmp print_string

done_print:

ret

msg_start: db 'Start Delay...', 13, 10, 0

msg_end:   db 'End Delay!', 13, 10, 0

Step 2: Compile and Run

- Assemble: nasm delay_test.asm -o delay_test.com

- Run in your emulator/DOSBox.

- Observe the pause between "Start Delay..." and "End Delay!"

Task 1: The "5-Second" Calculation

Problem: You are working on an embedded system using an 8086 processor running at 5 MHz. The time per clock cycle is exactly 200 nanoseconds. You need to create a delay of exactly 5 seconds.

Using the nested loop structure below, calculate the value required for the CX (Outer Loop) register. Assume the inner loop structure takes roughly 200 cycles to complete one iteration of the middle loop.

Given:

- Inner Loop Iterations (DX) = Fixed logic (Cost = 200 cycles per Middle iteration)

- Middle Loop Iterations (BX) = 500

- Outer Loop Iterations (CX) = ?

Required:

- Calculate total cycles needed for 5 seconds.

- Calculate the value for CX.

- Write the assembly code for this specific delay subroutine.

Task 2: The Blinking "WARNING" Sign

Problem: Write an assembly program that displays the text "WARNING" in the center of the screen. The text must blink (appear and disappear) indefinitely until the user presses a key.

Requirements:

- Clear the screen.

- Print "WARNING" at a fixed location.

- Call a Delay Routine.

- Clear the screen (or overwrite the text with spaces).

- Call a Delay Routine.

- Repeat.

- Use a check for Key Press (Standard BIOS Interrupt INT 16h) to exit the loop.