Program: BS Computer Science

Semester: Fall 2025

Credit Hour(s): 1

Pre-requisite(s): EE1005 Digital Logic Design

Lab: Computer Organization and Assembly Language

Instructor: Engr. Muhammad Qasim – muhammad.qasim@nu.edu.pk

Lab # 6: Branching, Flags, Comparison and Conditions

1. Objectives

- Understand and use different conditional jump instructions.

- Implement unconditional jumps.

- Apply relative addressing mode in jump instructions.

2. Jumps

Jump instructions transfer program control to a different location in the code.

2.1. Flags Involved in Jumps

- ZF (Zero Flag)

- SF (Sign Flag)

- CF (Carry Flag)

- OF (Overflow Flag)

2.2. Unconditional Jump

The JMP instruction transfers control to a specified label unconditionally, without checking any flags.

Syntax:	JMP LABEL

2.3. Conditional Jumps

Conditional jumps check the flag register and only jump if the condition is met. They are grouped based on the type of comparison:

A. Based on Single Flags:

| Instruction | Condition | Description | Flags Checked |
| --- | --- | --- | --- |
| JE | Jump if Equal | Jumps if the operands of the preceding comparison were equal | ZF = 1 |
| JNE | Jump if Not Equal | Jumps if the operands were not equal | ZF = 0 |
| JNZ | Jump if Not Zero | Jumps if the result of the preceding operation was not zero | ZF = 0 |
| JC | Jump if Carry | Jumps if CF is set. | CF = 1 |
| JO | Jump if Overflow | Jumps if OF is set. | OF = 1 |
| JS | Jump if Sign | Jumps if SF is set (result is negative). | SF = 1 |
| JNS | Jump if No Sign | Jumps if SF is clear (result is non-negative). | SF = 0 |

B. Based on Signed Comparisons: (Used for signed integers)

| Instruction | Condition | Description | Equivalent to |
| --- | --- | --- | --- |
| JG | Jump if Greater | Jumps if destination > source | JLE (Jump if Not Less than or Equal) |
| JL | Jump if Less | Jumps if destination < source | JNGE (Jump if Not Greater than or Equal) |
| JGE | Jump if Greater or Equal | Jumps if destination ≥ source | JNL (Jump if Not Less than) |
| JLE | Jump if Less or Equal | Jumps if destination ≤ source | JNG (Jump if Not Greater than) |

C. Based on Unsigned Comparisons: (Used for unsigned integers)

| Instruction | Condition | Description | Equivalent to |
| --- | --- | --- | --- |
| JA | Jump if Above | Jumps if destination > source | JNBE (Jump if Not Below or Equal) |
| JB | Jump if Below | Jumps if destination < source | JNAE (Jump if Not Above or Equal) |
| JAE | Jump if Above or Equal | Jumps if destination ≥ source | JNB (Jump if Not Below) |
| JBE | Jump if Below or Equal | Jumps if destination ≤ source | JNA (Jump if Not Above) |

2.4. Relative Addressing in Jumps

In modern assembly, most jump instructions use relative addressing.

- Instead of jumping to a fixed or absolute memory address, the program jumps to an address relative to the current instruction pointer.

- Target address is calculated as:

Target = Current Instruction Pointer (IP) + Displacement (Offset)

- Displacement is usually 8-bit or 16-bit signed value, so jumps can go forward or backward.

Example:

JMP +10    	; Jump 10 bytes forward

JMP -5     	; Jump 5 bytes backward

JMP NEXT   	; CPU adds displacement to IP to reach NEXT

3. Example

Example 1: Loop Using Relative Jump

[org 0x0100]

mov cx, 5

mov ax, 0

start_loop:

add ax, 2

loop start_loop   ; Uses relative addressing

mov ax, 0x4c00

int 0x21

Example 2:

[org 0x0100]

mov ax, 5

mov bx, 3

mov cx, 0

mov dx, 0

cmp ax, bx

je  case_equal        ; Jump if Equal (ZF=1)

jne case_not_equal    ; Jump if Not Equal (ZF=0)

; AX=5, BX=3 → ZF=0 → goes to JNE

case_equal:

mov cx, 1111h         ; Marker for Equal

jmp continue

case_not_equal:

mov cx, 2222h         ; Marker for Not Equal

continue:			; Greater / Less comparisons (signed)

cmp ax, bx

jg  case_greater      ; Jump if Greater (signed)

jle case_less_equal   ; Jump if Less or Equal (signed)

case_greater:

mov dx, 3333h

jmp unsigned_check

case_less_equal:

mov dx, 4444h

; Unsigned comparisons

unsigned_check:

cmp ax, bx

ja  case_above        ; Jump if Above (unsigned >)

jb  case_below        ; Jump if Below (unsigned <)

case_above:

mov [result1], 55h

jmp sign_check

case_below:

mov [result1], 66h

; Sign flag test

sign_check:

mov ax, -2

test ax, ax

js  case_negative     ; Jump if Sign = 1

jns case_non_negative ; Jump if Sign = 0

case_negative:

mov [result2], 77h

jmp overflow_check

case_non_negative:

mov [result2], 88h

; Overflow flag test

mov al, 127

add al, 1             ; 127 + 1 → overflow (OF=1)

jo  case_overflow

jno case_no_overflow

case_overflow:

mov [result3], 99h

jmp end_prog

case_no_overflow:

mov [result3], 0AAh

; End Program

end_prog:

mov ax, 0x4c00

int 0x21

result1: db 0

result2: db 0

result3: db 0

4. Lab Tasks

Task 1: Write a program to compare two numbers stored in registers. If they are equal, store 1 at memory location num, otherwise store 0.

Task 2: Write a program that checks if a number is positive, negative, or zero using conditional jumps. Store results in memory.

Task 3: Write a program to implement a simple countdown from 5 to 1 using JNZ.

Task 4: Write a program that calculates the sum of numbers from 1 to 10 using JMP (unconditional jump for looping).